Q . Simulate decimation by a factor of 2 with anti-alias filtering and analyze the resulting signal

spectrum. (25) (BL3) (WK4)

Given Data:

● Sampling Frequency = 8 kHz

● Signal Frequencies = 500 Hz and 2 kHz

● Decimation Factor = 2

Tasks:

1. Design anti-alias filter.

2. Filter the signal.

3. Perform decimation.

4. Plot spectra before and after decimation.

5. Verify alias prevention.

6. Discuss observations.

Aim

To simulate decimation by a factor of 2 using an anti-aliasing filter and analyze the signal spectrum before and after decimation.

Introduction

Decimation is the process of reducing the sampling rate of a digital signal by an integer factor. Before downsampling, an anti-aliasing low-pass filter is applied to remove high-frequency components that could cause aliasing. In this experiment, a signal sampled at 8 kHz is filtered and then decimated by a factor of 2. The spectra before and after decimation are compared to verify that aliasing is successfully prevented.

Code

clc;

clear;

close all;

%% Given Data

Fs = 8000;          % Original Sampling Frequency (Hz)

F1 = 500;           % Signal Frequency 1 (Hz)

F2 = 2000;          % Signal Frequency 2 (Hz)

D = 2;              % Decimation Factor

Fs_new = Fs/D;      % New Sampling Frequency (Hz)

%% Time Vector

t = 0:1/Fs:0.02;    % 20 ms duration

%% Input Signal

x = sin(2*pi*F1*t) + sin(2*pi*F2*t);

%% 1. Design Anti-Alias Filter

% Cutoff frequency must be less than new Nyquist frequency

Fc = 1800;                          % Cutoff Frequency (Hz)

N = 60;                             % Filter Order

Wn = Fc/(Fs/2);                     % Normalized Cutoff

b = fir1(N,Wn,hamming(N+1));         % FIR Low-pass Filter

%% 2. Filter the Signal

x_filtered = filter(b,1,x);

%% 3. Perform Decimation

x_dec = downsample(x_filtered,D);

%% Time Vector After Decimation

t_dec = (0:length(x_dec)-1)/Fs_new;

%% 4. Spectrum Before and After Decimation

Nfft = 4096;

X = fftshift(abs(fft(x,Nfft)));

Xf = fftshift(abs(fft(x_filtered,Nfft)));

Xd = fftshift(abs(fft(x_dec,Nfft)));

f = linspace(-Fs/2,Fs/2,Nfft);

f_dec = linspace(-Fs_new/2,Fs_new/2,Nfft);

%% Plot Time Domain Signals

figure;

subplot(3,1,1)

plot(t,x,'b')

title('Original Signal')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

subplot(3,1,2)

plot(t,x_filtered,'r')

title('Filtered Signal')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

subplot(3,1,3)

plot(t_dec,x_dec,'k')

title('Decimated Signal')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

%% Plot Frequency Spectra

figure;

subplot(3,1,1)

plot(f,X/max(X),'b')

title('Spectrum Before Filtering')

xlabel('Frequency (Hz)')

ylabel('Normalized Magnitude')

grid on

xlim([-4000 4000])

subplot(3,1,2)

plot(f,Xf/max(Xf),'r')

title('Spectrum After Anti-Alias Filter')

xlabel('Frequency (Hz)')

ylabel('Normalized Magnitude')

grid on

xlim([-4000 4000])

subplot(3,1,3)

plot(f_dec,Xd/max(Xd),'k')

title('Spectrum After Decimation')

xlabel('Frequency (Hz)')

ylabel('Normalized Magnitude')

grid on

xlim([-2000 2000])

Result

The decimation by a factor of 2 was successfully simulated using an FIR anti-aliasing low-pass filter. The filter effectively removed high-frequency components before downsampling, preventing aliasing. The frequency spectrum after decimation showed that the desired signal components were preserved without spectral distortion, confirming the effectiveness of the anti-alias filtering process.

output